# IT8212 External BIOS History

| ## Version | ## History | ## Date |
| --- | --- | --- |
| 1.3.1.0 | - First release | 02/18/’03 |
| 1.3.1.1 | - Set feature and get proper timing for all the storage devices on IT8212. - Bug fix for Acer 40x CDROM. - Bug fixed: INT13h can’t be hooked on one Compaq PC. - If error bit still on after drive select, send command anyway. - Add 4 seconds delay before first time access HDDs. | 03/26/’03 |
| 1.4.1.2 | - Bug fixed: the drive letters of same channel’s master and slave drives are exchanged. - Return ax=110h instead of 100h when leaving BIOS initial routine. - Modify cable type detection. - Bug fixed: DMA transfer failure on certain main boards. | 06/09/’03 |
| 1.4.1.3 | - Always boot from 8212 HDD when using Phoenix BIOS | 07/22/’03 |
| 1.5.1.3 | - No change | 10/01/’03 |
| 1.6.1.41 | - Bug fixed: for Gigabyte VIA chipset MB, fail to boot from secondary HDD. - Bug fixed: bootable CD cannot boot on some main boards and CD-ROMs. | 02/25/’04 |
| 1.7.1.51 | - Shorten devices scan time. - Hard disks attached on IT8212 can be shown in system BIOS’s hard disk boot priority list. | 06/02/’04 |
| 1.7.1.55 | 1. Bug fixed: returned wrong DPTE segment at offset 26. 2. Modify BCV Entries and EnableIRQ routine 3. Show bootable CDROM drive NO. 4. Fix some device not found | 11/25/’04 |
| 1.7.1.56 | 1. Bug fixed: force to mapping to 3000 but not setting ds 2. 懷疑是on-board的IDE BIOS code用過 40:8e (hard disk interrupt control flag)後未清除，導致IT8212誤判Interrupt已產生，但實際上DMA transfer 未完成 | 12/28/’04 |
| 1.7.1.57 | 1. Can show CDROM model name on AMI BIOS boot priority. 2. fix the values of base class/subclass/program interface in PnP expansion header and PCI header 3. show all the CDROMs with bootable CDs in system BIOS's boot priority | 03/25/’05 |
| 1.7.1.58 | 1 Recalculate last PnP expansion header's check sum after clear BCV Entries. 2. PCI header image length update 3. delay for BENQ 652A and Plextor CD-ROM 4. shorten CDROM detect time without CD | 04/20/’05 |
| 1.7.1.59 | - Change CDROM detects using RequestSenseKey Func. - Fix LG CD/RW not found - Turn the BEV on for support the AMI and AWARD BIOS boot priority issue. | 07/18/’05 |