NDIS Driver Program Installation             March 1993

================================

The NDIS driver programs in this disk support all DE-220 series Ethernet

cards.

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Directory       Contents

---------       -----------------------------

\NDIS\DOS       NDIS driver for DOS

\NDIS\OS2       NDIS driver for OS/2

D-Link NDIS driver program for DOS follows the Microsoft's NDIS spec v2.0.1.

D-Link NDIS driver program support the following network operating systems:

MS LAN Manager v2.1

DECnet PathWork v4.0

WIN/TCP PathWay Access for DOS v1.1

3COM 3+ Open v1.1

Banyan VINES v4.00(2)

IBM LAN Support Program v1.2.1

Windows for Workgroups v3.11

IBM OS/2 Lan Server v2.0

IBM LAN SERVER v3.0

IBM Communication Manager v1.0

***********************************************

*  How to install DE220.DOS on MS LAN MANAGER *

***********************************************

1)  Prepare a diskette with volume name "DOS_DRV_DLK" and create directories

in the following manner:

>label a: DOS_DRV_DLK     --insert a blank disk in drive 'A'

>md MSLANMAN.DOS          --make a directory called MSLANMAN.DOS

>cd MSLANMAN.DOS

>md DRIVERS               --make a directory called DRIVERS

>cd DRIVERS

>md ETHERNET              --make a directory called ETHERNET

>md NIF                   --make a directory called NIF

>cd ETHERNET

>md DE220                 --make a directory called DE220

MSLANMAN.DOS  (root)

|

DRIVERS

________|_________

|                  |

ETHERNET            NIF

|

-------------

|           |

DE220

2) Insert the DE-220 driver program diskette in drive 'A' and insert the

prepared diskette (DOS_DRV_DLK) in drive 'B'.  Afterwards, copy the

appropriate NDIS driver files from drive 'A' to 'B'.

>a:                     --go to drive 'A'

>cd \ndis\dos

>copy DE220.DOS     B:\MSLANMAN.DOS\DRIVERS\ETHERNET\DE220

>copy PROTOCOL.INI  B:\MSLANMAN.DOS\DRIVERS\ETHERNET\DE220

>copy DE220.NIF     B:\MSLANMAN.DOS\DRIVERS\NIF

3) MS LAN Manager provides a SETUP.EXE program on their Setup disk.

Execute this SETUP program to specify the proper driver for DE220 card.

-If you are installing LAN Manager for the first time, do the following:

Upon completion of retrieval of the last workstation's disk contents,

you will be asked if you want to import additional network drivers.

Answer "YES".

-If you have already installed LAN Manager in your harddisk, do the

following:

Select the Configuration menu, and then select "Add New Config" option.

Afterwards, choose the "Other Driver" option.

4) Insert the prepared diskette (DOS_DRV_DLK) in drive 'A' and follow the

instructions on the screen.  The SETUP program will copy the necessary

NDIS driver programs from the DOS_DRV_DLK diskette to your harddisk (into

the LAN Manager directory).

5) After completion of the SETUP, your CONFIG.SYS file will be modified and

contain statements referring to the D-Link NDIS driver programs.

Reboot your computer and follow the NDIS binding procedure to use the D-Link

NDIS driver programs.

*******************************************************

*  How to install DE220.DOS on WIN/TCP PathWay Access *

*******************************************************

1) Copy DE220.DOS to directory C:\LANMAN.

2) Go to the directory C:\LANMAN, and create a text file called PROTOCOL.INI:

[DE220]

drivername = DE220$

3) Add the following statements to your CONFIG.SYS file:

device=c:\lanman\DE220.DOS

4) Add the following statements to your AUTOEXEC.BAT file:

ndis

winbind

wintcp

****************************************************

* How to install DE220.DOS on DECnet PathWork v4.0 *

****************************************************

You can use the DECnet PathWork's  NETSET.EXE program to install D-Link NDIS

driver program. Please refer to DECnet installation manual for detailed

information. The following instructions assume that you have already installed

the DECnet PathWork with 3Com's Etherlink II card.

1) Modify your CONFIG.SYS file as follows:

device = c:\decnet\ELNKII.SYS

to

device = c:\decnet\DE220.DOS

2) Modify the PROTOCOL.INI file as follows:

from:

---------------------------------

[ELNKMACII]

drivername = ELNKII$

ioaddress = 0x300

interrupt = 3

xmitbufs = 2

[IPX4MAC]

:

bindings = ELNKMACII

:

[DATALINK]

:

bindings = ELNKMACII

:

to:

-----------------------------------

[DE220]

drivername = DE220$

[IPX4MAC]

:

bindings = DE220

:

[DATALINK]

:

bindings = DE220

:

********************************************

* How to install DE220.DOS on 3Com 3+ Open *

*********************************************

Supposing that you have installed the 3Com 3+ Open with 3Com's 3C501 card:

1) Copy DE220.DOS to  C:\3OPEN\DOSWKSTA\LANMAN\DRIVERS.

2) Modify the device driver statement for 3C501 card in your CONFIG.SYS to:

device = c:\3open\doswksta\lanman\drivers\DE220.DOS

3) Modify the PROTOCOL.INI file to:

[ETHERLINK]

:

drivername = DE220$

:

*****************************************************

* How to install DE220.DOS on Banyan VINES v4.00(2) *

*****************************************************

NOTE : To install D-Link NDIS driver program, you need Banyan VINES v4.00(2)

or above.

1. Create a bootable diskette (booted with DOS 3.1 or above) which will be

used as boot diskette of Banyan VINES workstation or create a sub-directory

on your hard disk.

There are two methods to copy the required files for Banyan VINES workstation

to the bootable diskette or sub-directory of hard disk.

Method 1.

1) Execute 'INSTALL' command which was saved in the diskette labeled

"VINES MASTER DISK".

2) Follow the instructions to copy the files to the bootable diskette

or sub-directory of hard disk.

Method 2.

1) Execute 'BAN' command.

2) Login VINES system.

3) Execute 'PCCOPY' command.

4) Select 'NDIS Ethernet' item and follow instructions as menu driven

for copying the files to the bootable diskette or sub-directory of

hard disk.

2. Insert the DE-220 driver program diskette in drive 'A' and insert the

prepared bootable diskette in drive 'B'. Afterwards, copy the appropriate

NDIS driver files DE220.DOS from drive 'A' to 'B' or sub-directory of hard

disk.

3. Create a CONFIG.SYS file which contains the following statements:

DEVICE=PROTMAN.DOS /I:A:\

DEVICE=DE220.DOS

NOTE:" A:\" is the path which may be changed depending on the location

of the files for Banyan VINES workstation.

4. Create a DE220.NIF file and include the following statements in this file:

Model = D-Link DE-220 Ethernet Card

Path = A:\

DeviceDriver = DE220.DOS

DeviceName = DE220$

Type = NDIS

NOTE:" A:\" is the path which may be changed depending on the location

of the files for Banyan VINES workstation.

5. Create a PROTOCOL.INI file as follows:

[PROTOCOL MANAGER]

drivername = PROTMAN$

[VINES_XIF]

drivername = NDISBAN$

bindings = "DE220"

[DE220]

drivername = DE220$

6. Execute 'PCCONFIG' command

1) The "CONFIGURING A PC FOR THE NETWORK" is displayed on the screen

then select item '1 - Network Card Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

The "NDIS Ethernet" is displayed on the screen

then set your choice for D-LINK NIC

Ex. Interrupt Level (2-15) : 3

PROTOCOL.INI bindings = DE220

|

2) Return to Screen "CONFIGURING A PC FOR THE NETWORK"

then select item '2 - Login Environment Settings'.

|

The "Network Card Setting" is displayed on the screen

then select item 'NDIS Ethernet'.

|

then 'PCCONFIG.DB' file will be generated.

7. Reboot your computer.

8. Login into VINES by executing the 'BAN' command.

**************************************************************

* How to install DE220.DOS on IBM LAN Support Program v1.2.1 *

**************************************************************

Supposing that you have installed the IBM LAN Support Program in your computer,

you can use DE220.DOS to replace the installed driver program. Follow the

instructions below to change the contents of CONFIG.SYS and PROTOCOL.INI files.

1) Copy DE220.DOS to the directory where your IBM LAN Support program resides.

2) Modify the original NDIS device driver statement in CONFIG.SYS as follows:

DEVICE = [driver]:\path\DE220.DOS

3) Modify the PROTOCOL.INI file as follows:

BINDINGS = DE220

:

:

[DE220]

DRIVERNAME = DE220$

***********************************************************

* How to install DE220.DOS on Windows for Workgroups v3.1 *

***********************************************************

There are two methods for installing DE-220 using Windows for

Workgroups v3.1

+---------------------------------------+

|  Install a compatible network adapter |

+---------------------------------------+

Using a compatible driver that is provided by Microsoft.  The

following table correlates the drivers that is provided by

Microsoft to the various compatible D-Link adapters.

Microsoft drivers                D-Link network adapters

-------------------------------  --------------------------------

IBM Token Ring 4/16Mbs           DT-100, DT-220

IBM Token Ring 4/16Mbs (MCA)     DT-300

NE1000 Compatible                DE-100

NE2000 Compatible                DE-200

DE-205TP

DE-220 series (8 or 16-bit slot)

SMC ARCNET PC100                 DA-100, DA-100+

PC200                 DA-200, DA-200+

-------------------------------  --------------------------------

Installation guide:

a. Proceed to install Windows for Workgroups.  If you already have an

installed Windows for Workgroups, open the Control Panel Window to

change the network settings.

b. When Windows asks for the type of network adapter, select the

appropriate network adapter following the table provided above.

c. Input the corresponding settings such as IRQ and I/O port address.

After checking all the settings, choose OK and restart Windows for

Workgroups.

Method I:

Manual editing of pre-selected configuration

Involves manual editing of installed Windows for Workgroups

configuration files using pre-selected NE2000 driver.

Please refer to WFW1.DOC for detailed installation procedures.

Method II:

Menu-driven installation

Using a D-Link supplied OEMSETUP.INF file, the user can choose to

install the appropriate D-Link adapters through a menu.

Please refer to WFW2.DOC for detailed installation procedures.

********************************************************

* How to install DE-220.OS2 on IBM OS/2 2.0 Lan Server *

********************************************************

1. Assuming your OS/2 2.0 is properly installed and working, proceed to

install LAN SERVER.  At this point in time, no D-Link network adapters

must be installed in the computer.

2. Install LAN SERVER according to IBM installation procedures.  At the

point where LAN SERVER asks for LAN Adapter, select any of the adapters

supported by LAN SERVER.  For example, we select 'Western Digital

EtherCard PLUS ...'.  After LAN Adapter selection, continue installing

LAN SERVER.

3. After LAN SERVER is successfully installed, shut down OS/2 for system

econfiguration.  Do not insert any D-Link network adapters.  The OS/2

system will boot up bringing up LAN SERVER services. You will encounter

errors with LAN SERVER because the network adapter you selected is not

installed.  Ignore the errors and continue with OS/2 booting.

4. Assuming you are using D-Link DE-220 card, insert the DE-220 driver disk

in drive A: and copy the following files,

COPY A:\NDIS\OS2\DE220.OS2 C:\IBMCOM\MACS

COPY A:\NDIS\OS2\DE220.NIF C:\IBMCOM\MACS

At the OS/2 Desktop, run OS/2 system editor.  You have to edit two files

namely,

C:\CONFIG.SYS

C:\IBMCOM\PROTOCOL.INI

Editing CONFIG.SYS

------------------

(1) Look for the following line

DEVICE=C:\IBMCOM\MACS\MACWD.OS2

and change it to,

DEVICE=C:\IBMCOM\MACS\DE220.OS2

Editing PROTOCOL.INI

--------------------

(1) In the section [IBMLXCFG], add the following line

DE220_nif = DE220.nif

(2) In the section [NETBEUI_nif], look for the line

Bindings = MACWDMC_nif

and change it to,

Bindings = DE220_nif

(3) In the MAC SECTION definitions, add the following section

[DE220_nif]

DriverName = DE220$

5. After modifying the two files above, shutdown your OS/2 system and

turn the computer off.  Install the D-Link DE-220 card using the same

INTERRUPT and IOADDRESS specified in the PROTOCOL.INI file.  Turn on

your computer to start OS/2.

Upon OS/2 startup, you should notice that LAN SERVER services will now

successfully start without any errors.

**************************************************

* How to install DE220.OS2 on IBM LAN SERVER 3.0 *

*************************************************

1. Prepare an empty disk and insert it into drive 'A'.  Afterwards,

insert the D-Link Drive disk in drive 'B' and do the following:

For an OS2 machine, you need to use the OS2 NDIS driver.

> copy B:\NDIS\OS2\DE220.OS2     A:\

> copy B:\NDIS\OS2\DE220IBM.NIF  A:\DE220.NIF

OR, for a DOS machine, you need to use the DOS NDIS driver.

> copy B:\NDIS\DOS\DE220.DOS          A:\

> copy B:\NDIS\DOS\DE220IBM.NIF  A:\DE220.NIF

** The DE220IBM.NIF file must be renamed to DE220.NIF

2. You must first install IBM OS/2 in your system:

Bootup with OS/2 Install Disk in drive 'A' and install OS/2

You must install the LAPs program before installing the LAN Server

or Requester program. Insert the "NTS/2 Adapter & Protocol Support Disk"

in drive B and run the LAPs program:

>LAPS  --choose "Install" and specify the target drive

choose "Install" again and insert your prepared

Driver disk in drive 'A'

choose "Configure"

click "D-Link DE-220 Ethernet Card" --> Add

click "IBM OS/2 NETBIOS"            --> Add

click "IEEE 802.2"                  --> Add

click OK  --> Exit  -->  Continue

Reboot your computer

Your CONFIG.SYS and PROTOCOL.INI files should reflect your card setup:

\CONFIG.SYS

===========

:

Device=d:\IBMCOM\MACS\DE220.DOS  or DE220.OS2

:

\IBMCOM\PROTOCOL.INI

====================

:

[DE220_NIF]

Drivername = de220$

3. You can now install the LAN Server or Requester program.  For more

instruction, please refer to your IBM LAN Server "Planning and

Installation" manual.

***************************************************************

* How to install DE220.OS2 on IBM Communication Manager v 1.0 *

***************************************************************

Supposing that you have installed the IBM Communication Manager in your

computer.

1. Copy both DE220.OS2 and DE220.NIF from the \NDIS\OS2 subdirectory of

the shipping driver diskette to C:\IBMCOM\MACS directory.

2. Execute CM's install program, select D-Link DE-220 adapter and select

protocols.

3. Make sure "DEVICE = C:\IBMCOM\MACS\DE220.OS2" statement is in the

CONFIG.SYS file.

4. Shutdown your OS/2 system, then reboot your PC.

==============================================================================

D-Link NDIS PROTOCOL.INI User Guide

The keywords used to create the PROTOCOL.INI file for DE220 is as follow:

DRIVERNAME -This specifies the network card's driver name.

**Following is an example of a PROTOCOL.INI file that can be used with D-Link

DE220 series Ethernet card:

Drivername = DE220$