November 1992

SCO UNIX V.3 Driver Installation

for

D-Link DE-220 Series Ethernet Card

==================================

The following files will be used in this installation:

Files	       Contents

-----------    -----------------------------------------

de220.o        DE-220 Driver Program

dlk

tcp

1. Login as the root user.

2. Installing TCP/IP software through the custom utility.

# custom

Select install

Select A New Product

Select entire to install all of the packages

3. Rename /usr/lib/mkdev/tcp to /usr/lib/mkdev/tcp.save if you want to keep

original /usr/lib/mkdev/tcp file. (it will be modified by next step)

4.  Installation diskette in Dos format

Put your D-Link Installation Diskette to floppy A and use doscp command

to copy D-Link SCO UNIX Driver to following directories.

# cd /

# mkdir /usr/lib/tcprt/dlk

# doscp A:/scounix/tcpip1.1x/readme.doc /usr/lib/tcprt/dlk/readme.doc

# doscp A:/scounix/tcpip1.1x/de220.o /usr/lib/tcprt/dlk/de220.o

# doscp A:/scounix/tcpip1.1x/dlk /usr/lib/mkdev/dlk

# doscp A:/scounix/tcpip1.1x/tcp /usr/lib/mkdev/tcp

# chmod +x /usr/lib/mkdev/dlk

# chmod +x /usr/lib/mkdev/tcp

5. Installing D-Link SCO UNIX Driver

# mkdev dlk

Do you wish to install or delete the dlk driver?  type i to install

A display of information about driver.		  type y to continue

Select D-link Ethernet adapter			  select de220

Select interrupt vector 			  value

Select I/O base address 			  hexadecimal value

Select memory base address			  hexadecimal value

Do you want relink the kernel?			  type y to relink

Do you want this kernel to boot by default?	  type y to default

Do you want the kernel environment rebuilt?	  type y to rebuilt

6. Configuring Network interface

# mkdev tcp

which driver(s) will TCP/IP be using?		select dlk

DOMAIN name					enter key

IP address					n.n.n.n

broadcast address of all 0's?                   [y/n]

confirm broadcast address			enter key

confirm netmask 				enter key

which driver(s) will TCP/IP be using?		quit

7. Reboot unix

# shutdown -y -g0